Materials

450” of 1.25” aluminum angle 0.125” (1/8”) wall thickness

36” of 1” wide flat aluminum strap 0.0625” (1/16”) thickness

26” of 3.5” O.D. PVC pipe

12” of piano hinge for door

Pull knob for door

Light chain for door support

S-hooks for door support

Two Pulleys to accommodate rope

3 ft of ½” Rope

#10 machine bolts and nuts

#10 rubber screw protectors

(2) 0.5 inch Eye bolts

(7) 0.118” thick X 48” x 24” Clear Polycarbonate sheets

5.5” of 2” round bar to make a 10 lb. weight to drop through PVC (may substitute material)

Caulk to seal PVC pipe to flange

(4) Rubber chair glides  for base

Tools

Drill

Drill bits

Screw driver

Wench

Saw for cutting metal

Cut list

(2) 49” of angle for front upright supports

(4) 17.25” of angle for vertical members of lower box section

(1) 4” of angle for brace support for lower section

(8) 16.5” of angle for horizontal member of lower box section

(2) 32” of angle for vertical members for the back of the upper section

(2) 16.5” of angle for cross supports to connect vertical members of upper section

(1) 16.5” of angle for front upper cross member

(1) 4” of angle for mounting flat strap to rear cross member.

Side View

Top view

(1) 21” of flat strap for support around PVC pipe

Top ViewSide view

(1) 8” of flat strap for cross support between vertical uprights

PVC Polycarbonate Flange

Drop Weight

Hole for nail

Hole for set screw